SMART INDIA HACKATHON (SIH) — INERTIAL DEAD RECKONING

Project Timeline & Engineering Evolution

Complete Chronological Engineering Record: Phase 0 Inception to Phase 9 Production Flagship

📈 ENGINEERING TRAJECTORY: DIVERGING BASELINE TO SUB-10% PRODUCTION ENGINE This document chronologically details each engineering phase, hypotheses tested, failure modes encountered, root-cause telemetry discoveries, and mathematical solutions that reduced dead-reckoning drift from >700% down to 1.9%–9.7% across all tunnel scenarios.

1. High-Level Timeline Architecture

PHASE 0: Inception, Problem Formulation & IO-VNBD Data Engineering │ └─ Smartphone sensor constraints (no CAN-bus/OBD-II), Oxford IO-VNBD dataset ▼ PHASE 1: Classical Inertial Navigation Baseline & The Physics of Divergence │ └─ Double integration ∫ a dt; bias growth O(t²); exploded to >200 km/h in 32s (>700% drift) ▼ PHASE 2: IDNN v1 — First Deep Learning Architecture (Tapped Delay Lines) │ └─ 20 tapped delay taps (126 inputs); negative speeds, tilt vulnerability (30%–56% drift) ▼ PHASE 3: IDNN v2 — Spectral Feature Engineering & The Overfitting Trap │ └─ 10-pt FFT spectral bands, rolling stats, BatchNorm; memorized vehicle suspension frequencies ▼ PHASE 4: IDNN v3 — 100% Blind Cross-Drive Training, Kinematic Jerk & Kalman Filter │ └─ Strict external training (Pairs 1-7); 1D KF smoother + ZUPT; mountain tunnel: 5.8% ▼ PHASE 5: IDNN v4 — Physics-Gated Loss, Attitude Decoupling & Acceleration Feedforward │ └─ Dynamic gravity projection ¦¦a_h¦¦, suspension pitch θ_pitch, plausibility gate, cruise lock ▼ PHASE 6: IDNN v5 — Vehicle Idle Normalization & Online Pre-Blackout GNSS Calibration │ └─ Mount-invariant vibration scaling σ_idle; real-time pre-blackout scale/bias estimator ▼ PHASE 7: The "Medium Tunnel" Crisis & Root-Cause Telemetry Diagnostics (66.2% → 9.7%) │ └─ Pre-entry braking lag freezing + 5° incline standstill failure; solved via Braking Guard & AC ZUPT ▼ PHASE 8: 2D Spatial Propagation, Road Map-Matching & Fixing 0.0m Interpolation Artifact │ └─ Heading analysis; road polyline snapping; 1,000-sample extension past exit ▼ PHASE 9: Publication-Grade Multi-Model Benchmarks & Executive Deliverables └─ Unified all-models comparison suite, Master Engineering Report (.md, .html, .docx)

2. Chronological Phase-by-Phase Breakdown

Phase 0: Project Inception, Problem Formulation & Data Engineering

The objective was established: build a robust vehicular dead-reckoning navigation engine that maintains continuous positioning during GNSS outages (tunnels, underpasses, urban canyons) operating strictly on consumer smartphone sensors (accelerometer, gyroscope, magnetometer, gravity). No CAN-bus, no OBD-II dongle, no external hardware.

Dataset (Oxford IO-VNBD): Over 100+ km of driving across diverse vehicle models, mounts, and road conditions.

Ground Truth: Centimeter-accurate dual-frequency RTK-GNSS and tactical reference odometry sampled at 10 Hz (Δt = 0.1s).

Core Code Created: src/data_loader.py for automated column mapping, timestamp synchronization, and linear interpolation.

Phase 1: Classical Inertial Navigation Baseline & The Physics of Divergence

Textbook Newtonian double integration was implemented: v(t) = v₀ + ∫ a_long dt, s(t) = s₀ + ∫ v dt.

Physics Discovery: Smartphone MEMS sensors have residual DC biases (0.05 to 0.20 m/s²) and gravity leakage (1° tilt leaks 0.171 m/s²). Integrating bias over time produces linear velocity error Δv = b_a·t and quadratic position error Δs = ½·b_a·t².

Empirical Failure: Baseline speed exploded past 200 km/h within 32 seconds while the vehicle was stationary in traffic!

Outage Drift: In a 60-second tunnel, baseline INS accumulated 3,284 meters of error (733% drift).

Figure T1: Classical Inertial Navigation Quadratic Divergence vs Deep Learning Speed Tracking (0 to 300s)

Phase 2: IDNN v1 — First Deep Learning Architecture (Tapped Delay Lines)

Hypothesis: An Input Delay Neural Network (IDNN) can learn the nonlinear mapping from sensor motion directly to forward vehicle speed, bypassing explicit accelerometer integration.

Why not LSTM? LSTMs suffer from 'memory poisoning'—a single bump corrupts the hidden state indefinitely. An IDNN uses a finite tapped delay line (20 taps = 2.0s history), guaranteeing zero long-term memory corruption.

Input: 6 raw channels (a_x, a_y, a_z, ω_x, ω_y, ω_z) × 21 taps = 126 inputs. Dense layers: 128 -> 64 -> 32 -> 1.

Failure Modes: Output head predicted non-physical negative speeds (v < 0); model lacked gravity awareness (30% to 56% drift).

Phase 3: IDNN v2 — Feature Engineering & The Overfitting Trap

Hand-engineered 33 features: rolling statistics (mean, std, RMS), 10-point FFT spectral energy bands (0-1 Hz, 1-3 Hz, 3-5 Hz), and BatchNorm1d layers.

🚨 THE IN-SAMPLE OVERFITTING TRAP When evaluated on the same route it trained on, IDNN v2 achieved an impressive RMSE of 2.29 m/s. However, when evaluated on a different vehicle, the model collapsed catastrophically. The FFT spectral bins had memorized the mechanical resonant frequency of the training vehicle's suspension and tire tread!

Phase 4: IDNN v3 — 100% Blind Cross-Drive Training, Kinematic Jerk & Kalman Filter

Strategic Shift: Strict Cross-Drive Protocol. Trained on 7 independent external drives (Pairs 1-7: S1..S4, Vfa01), validated on Vfa02, and tested on unseen 105 km Drive M.

Kinematic Jerk: First-order time derivatives of acceleration and angular rates (Δa/Δt, Δω/Δt) to detect coasting transitions.

Filtering: 1D Kalman Filter speed smoother (Q = 0.5, R = 4.0) and classical Zero-Velocity Update (ZUPT).

Results: Long Mountain Tunnel reached 5.8% drift, but underpasses remained high at 42.2% drift. ZUPT failed on inclined roads.

Phase 5: IDNN v4 — Physics-Gated Loss, Attitude Decoupling & Acceleration Feedforward

Integrated user-driven physical constraints:

Dynamic Gravity Decoupling: Projected specific force onto unit gravity to isolate horizontal acceleration norm ||a_h||.

Suspension Pitch: θ_pitch = arctan(g_x / √(g_y² + g_z²)) to compensate for suspension pitch during braking.

Kinematic Plausibility Gate: Clamped non-physical acceleration jumps: -6.0 to +3.5 m/s².

Acceleration Feedforward EKF: Direct accelerometer integration during hard braking to prevent neural delay lag.

Remaining Bottleneck: Highway tunnel drift improved to 12.8%, but mount stiffness differences across cars introduced an uncalibrated transfer scale offset.

Phase 6: IDNN v5 — Vehicle Idle Normalization & Online Pre-Blackout GNSS Calibration

Introduced two flagship architectural breakthroughs:

1. Idle Normalization: Samples engine idle baseline σ_idle while stationary with GPS; normalizes road vibration E_vib / σ_idle, making the model invariant to mount stiffness.

2. Online Pre-Blackout Calibrator: Continuously tracks (v_model - v_gnss) during healthy GPS; freezes the learned bias at blackout entry: v_calibrated = max(0, v_model - Bias_entry).

Milestone: Highway Tunnel dropped to 1.9%, Underpass to 2.8%, Mountain Tunnel to 5.1%!

Phase 7: The 'Medium Tunnel' Crisis & Root-Cause Telemetry Diagnostics (66.2% → 9.7%)

While other tunnels passed, Medium Tunnel (s_idx = 35000, 60s) spiked to 66.2% drift (296.5 m error).

Root Cause 1 (Braking Lag): Driver braked hard into tunnel (14.4 -> 12.7 m/s); filter lag created an apparent difference of -3.56 m/s, which the naive calibrator froze as an additive bias, inadvertently adding +12.8 km/h inside the tunnel.

Root Cause 2 (Standstill on Incline): Car stopped for 10s on a 5° road grade. Gravity tilted into the longitudinal axis (0.854 > 0.25 m/s²), preventing classical ZUPT from triggering. The car integrated +12.8 km/h while standing still (+213.6m false distance).

🛡️ DUAL KINEMATIC SOLUTIONS 1. Braking-Lag Kinematic Guard: Detects pre-entry braking (Δv < -1.0 m/s) and clamps negative lag bias to 0.0 m/s. 2. Road-Grade Invariant AC Vibration ZUPT: Evaluates high-frequency AC variance Var(¦¦a¦¦ - ¦¦g¦¦) < 0.035 m²/s⁴. When stopped on an incline, vibration variance drops to 0.0064 m²/s⁴, immediately zeroing speed regardless of road slope! RESULT: Medium Tunnel drift plummeted from 66.2% to 9.7%!

Figure T2: Medium Tunnel (60s Blackout, 448m) 2D Map-Matched Trajectory Showing Accurate Standstill Recovery

Phase 8: 2D Spatial Propagation, Road Map-Matching & Fixing 0.0m Artifact

Compared Gyro Integration (>300% 2D error), Compass (distorted by rebar), Heading Vector Lock (good for straight roads, fails on curves), and Road Polyline Frenet Frame Snapping.

Fixed 0.0m interpolation artifact by extending ground truth arrays 1,000 samples (1.5 km) beyond the tunnel exit, guaranteeing true un-clamped Euclidean exit errors.

Figure T3: 1D Speed Integration Drift vs 2D Map-Matched Exit Drift (All 5 Blackout Scenarios)

Phase 9: Publication-Grade Multi-Model Benchmarks & Executive Deliverables

Generated unified 4-figure comparison suite and 8 publication blackout simulation figures.

Produced master documentation: SIH_IDR_MASTER_PROJECT_DOCUMENT (.md, .html, .docx).

3. Quantitative Milestone Progression Matrix

| Milestone / Model | Speed RMSE | Highway Tunnel (30s) | Short Underpass (30s) | Medium Tunnel (60s) | Mountain Tunnel (120s) | SIH Target Status |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline INS (Double Integ.) | 44.72 m/s | 121.8% | 431.4% | 733.0% | 355.5% | FAIL (Diverged) |
| IDNN v1 (Simple MLP) | 3.80 m/s | 39.0% | 56.2% | 28.7% | 16.0% | FAIL (> 10%) |
| IDNN v2 (In-Sample Overfit) | 2.29 m/s | 32.6% | 13.4% | 1.6% | 2.8% | FAIL (Cross-Drive) |
| IDNN v3 (KF + ZUPT) | 3.44 m/s | 24.6% | 42.2% | 15.0% | 5.8% | PARTIAL (<10% Long) |
| IDNN v4 (Physics-Gated) | 3.94 m/s | 12.8% | 38.9% | 22.2% | 7.9% | PARTIAL (<10% Long) |
| IDNN v5 Raw Neural | 4.24 m/s | 13.3% | 32.2% | 11.1% | 7.4% | PARTIAL (<10% Long) |
| IDNN v5 Flagship (Calibrated) | 4.24 m/s | 1.9% 🏆 | 2.8% 🏆 | 9.7% 🏆 | 5.1% 🏆 | PASSED (< 10% ALL!) |

Figure T4: Quantitative Evolution of Along-Track Drift Percentage Across Model Generations

Figure T5: Comprehensive Multi-Model Benchmarking Matrix & Drift Verification

4. Key Engineering Lessons Learned

Double integration of MEMS acceleration always diverges quadratically O(t²). Machine learning speed estimation is mandatory.

In-sample route evaluation is deceptive; models overfit to vehicle mechanical frequencies. 100% blind cross-drive validation is non-negotiable.

Normalizing vibration energy by pre-drive engine idle variance σ_idle eliminates car-to-car mount stiffness discrepancies.

Healthy pre-blackout GPS should be utilized to learn instantaneous mount transfer bias, freezing it upon entry.

Deceleration into a tunnel must be guarded; otherwise causal filter lag is mistakenly frozen as positive speed bias.

Zero-velocity detection on road inclines requires AC vibration variance, which is invariant to static gravitational tilt.